# Literature Review: Illegal Mining Detection Using Satellite Imagery and Deep Learning

| S. No |  | Problem Statement | Methodology | Results | Shortcomings / Future Work |
| --- | --- | --- | --- | --- | --- |
| 1 |  | Lack of large-scale systems to detect illegal mines and tailings dams. | Fully Convolutional Neural Networks trained on Sentinel-2 imagery using Google Earth Engine. | Discovered 263 unregistered mines at country scale. | False positives in dam-like structures; needs region-specific tuning. |
| 2 |  | Difficulty in detecting fine-scale gold mining changes over time. | ReCNN (CNN + LSTM) using multi-temporal Sentinel-2 images. | High accuracy change detection of mining ponds. | Requires labeled temporal data; computationally intensive. |
| 3 |  | Mining-induced subsidence hard to distinguish from other deformation. | SBAS-InSAR with DeepLabv3+ CNN on deformation maps. | Detected 202 subsidence areas including forested zones. | Relies on SAR data quality; region-specific training. |
| 4 |  | Manual monitoring of open-pit mines is inefficient. | Improved Mask R-CNN with transfer learning on high-resolution images. | Accurate open-pit mine boundary detection. | High compute cost; not designed for dense forests. |
| 5 6 7 8 9 10 | Illegal aggregate mining difficult to monitor at large scale. Accurate detection of mining subsidence basins using radar data. Wide-area automatic detection of mining subsidence regions. Need for accurate mining land-cover classification. Detection of deforestation linked to mining activities. Monitoring illegal river dredging transport activities. |  | Transfer learning with DenseNet and ResNet on satellite images. Sentinel-1A SAR; D-InSAR; CNN feature extraction (AlexNet, VGG19, ResNet50) with AFSA-optimized SVM. InSAR deformation maps combined with Mask R-CNN for segmentation. High-resolution UAV imagery; U-Net convolutional neural network. Sentinel-2 imagery; DeepLabv3+ semantic segmentation. CNN-based object detection for truck recognition | Over 90% accuracy in detecting mining machinery. Achieved up to 90.6% detection accuracy. Improved large-area subsidence detection and boundary accuracy. Improved land-cover segmentation accuracy in mining areas. High accuracy in identifying deforested regions. Accurate real-time vehicle detection. | Detects machinery, not land deformation directly. Limited to subsidence detection; requires complex SAR preprocessing. Performance depends on deformation map quality. Requires UAV data; limited large-scale applicability. Focuses on deforestation, not direct mining detection. Camera-based system; not suitable for satellite-scale monitoring. |

## References

[1].  Balaniuk et al., Mining and Tailings Dam Detection in Satellite Imagery Using Deep Learning, Sensors, 2020.

[2].  Camalan et al., Change Detection of Amazonian Alluvial Gold Mining Using Deep Learning, Remote Sensing, 2022.

[3].  Xi et al., Automatic Identification of Mining-Induced Subsidence Using Deep CNNs, 2023.

[4].  Wang et al., Automatic Identification of Open-Pit Mines Using Improved Mask R-CNN, Remote Sensing, 2020.

[5].  Yesilmen et al., CNN-Based Monitoring of Aggregate Mining, Case Studies in Construction Materials, 2022.

[6].  Automatic-Detection Method for Mining Subsidence Basins Based on InSAR and CNN-AFSA-SVM, Sustainability, 2022.

[7]. Mask R-CNN Network for Wide-Area Mining Subsidence Automatic Detection With InSAR Observations, IEEE Transactions on Geoscience and Remote Sensing, 2024.

[8]. U-Net Convolutional Networks for Mining Land Cover Classification Based on High-Resolution UAV Imagery, IEEE Access, 2020.

[9].  Deforestation Detection in the Amazon Using DeepLabv3+ Semantic Segmentation Model Variants, Remote Sensing, 2022.

[10].  Automated Sensing System for Real-Time Recognition of Trucks in River Dredging Areas Using CNN, Sensors, 2021.